# Supplementary Tables

Manuscript ID: electronics-4413038

Title: P3: Persistence-Aware Admission Control for DoS-Resilient BLE Resolvable Private Address Resolution

The tables below report deterministic simulator outputs. They do not constitute measured BLE controller timing, packet-loss, energy, or production-firmware evidence. The accompanying reviewer artifact archive contains the seed-level summaries, configurations, manifests, audit tables, and reproduction scripts.

# Supplementary Table S1

Supplementary Table S1. Complete long-window validation results for N = 512 and RL = 8 (1800-second runs; 300-second warmup; n = 3 seed means for each scenario/method pair).

| Scenario | Method | n | Attack/min | Attack AES/min | Legit rate | False defer |
| --- | --- | --- | --- | --- | --- | --- |
| heavy_flood | BudgetDoS | 3 | 10000 | 45192.00 | 0.5233 | 0.4767 |
| heavy_flood | P3-NoPersist | 3 | 10000 | 45198.72 | 0.5602 | 0.4403 |
| heavy_flood | P3-NoBudget | 3 | 10000 | 5035658.88 | 1.0000 | 0.0088 |
| heavy_flood | P3-NoCache | 3 | 10000 | 45192.00 | 0.5233 | 0.4774 |
| heavy_flood | StaticRL | 3 | 10000 | 5035658.88 | 1.0000 | 0.0000 |
| heavy_flood | ZephyrCache | 3 | 10000 | 5035658.88 | 1.0000 | 0.0000 |
| medium_flood | BudgetDoS | 3 | 1000 | 43874.88 | 0.5809 | 0.4191 |
| medium_flood | P3-NoPersist | 3 | 1000 | 44593.92 | 0.7554 | 0.2459 |
| medium_flood | P3-NoBudget | 3 | 1000 | 502353.60 | 1.0000 | 0.0072 |
| medium_flood | P3-NoCache | 3 | 1000 | 43874.88 | 0.5809 | 0.4211 |
| medium_flood | StaticRL | 3 | 1000 | 502353.60 | 1.0000 | 0.0000 |
| medium_flood | ZephyrCache | 3 | 1000 | 502353.60 | 1.0000 | 0.0000 |
| normal_capacity | BudgetDoS | 3 | 0 | 0.00 | 0.8814 | 0.1186 |
| normal_capacity | P3-NoPersist | 3 | 0 | 0.00 | 0.9855 | 0.0234 |
| normal_capacity | P3-NoBudget | 3 | 0 | 0.00 | 1.0000 | 0.0096 |
| normal_capacity | P3-NoCache | 3 | 0 | 0.00 | 0.8814 | 0.1388 |
| normal_capacity | StaticRL | 3 | 0 | 0.00 | 1.0000 | 0.0000 |
| normal_capacity | ZephyrCache | 3 | 0 | 0.00 | 1.0000 | 0.0000 |

# Supplementary Table S2

Supplementary Table S2. Paired modeled-observable privacy audit (20 paired seeds per load/filter condition). The external proxy contains defer rate, mean modeled delay, and p95 modeled delay.

| Load | Δ (s) | Method | Pairs | Initial audit | External-proxy AUC | Bootstrap 95% CI |
| --- | --- | --- | --- | --- | --- | --- |
| no_attack | 0 | P3-Persist | 20 | All matched | 1.000000 | 1.000000–1.000000 |
| no_attack | 0 | P3-NoPersist | 20 | All matched | 1.000000 | 1.000000–1.000000 |
| no_attack | 0 | BudgetDoS | 20 | All matched | 0.500000 | 0.500000–0.500000 |
| no_attack | 60 | P3-Persist | 20 | All matched | 0.500000 | 0.500000–0.500000 |
| no_attack | 60 | P3-NoPersist | 20 | All matched | 0.500000 | 0.500000–0.500000 |
| no_attack | 60 | BudgetDoS | 20 | All matched | 0.500000 | 0.500000–0.500000 |
| medium_flood | 0 | P3-Persist | 20 | All matched | 1.000000 | 1.000000–1.000000 |
| medium_flood | 0 | P3-NoPersist | 20 | All matched | 1.000000 | 1.000000–1.000000 |
| medium_flood | 0 | BudgetDoS | 20 | All matched | 0.500000 | 0.500000–0.500000 |
| medium_flood | 60 | P3-Persist | 20 | All matched | 0.500000 | 0.500000–0.500000 |
| medium_flood | 60 | P3-NoPersist | 20 | All matched | 0.500000 | 0.500000–0.500000 |
| medium_flood | 60 | BudgetDoS | 20 | All matched | 0.500000 | 0.500000–0.500000 |
| heavy_flood | 0 | P3-Persist | 20 | All matched | 0.500000 | 0.500000–0.500000 |
| heavy_flood | 0 | P3-NoPersist | 20 | All matched | 0.500000 | 0.500000–0.500000 |
| heavy_flood | 0 | BudgetDoS | 20 | All matched | 0.500000 | 0.500000–0.500000 |
| heavy_flood | 60 | P3-Persist | 20 | All matched | 0.500000 | 0.500000–0.500000 |
| heavy_flood | 60 | P3-NoPersist | 20 | All matched | 0.500000 | 0.500000–0.500000 |
| heavy_flood | 60 | BudgetDoS | 20 | All matched | 0.500000 | 0.500000–0.500000 |

Note: Every paired job used identical visible timestamp, RSSI, payload-length, RPA-value, repetition-pattern, load, and duplicate-filter inputs; only the offline identity label and normal resolution outcome differed. ‘All matched’ refers only to the tested initial pre-resolution decision. AUC values are simulator-derived modeled-proxy results and do not establish real end-to-end side-channel freedom. Full paired effects and internal mechanism diagnostics are supplied as CSV artifacts.

# Supplementary Table S3

Supplementary Table S3. P3-Persist joint RPA-epoch, modeled observation-loss, and benign-unknown boundary (20 seeds per cell).

| Cell | Epoch (min) | Loss | Benign/min | Benign mode | Offered legit | Observed legit | Observed false defer | Attack AES/min |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| main | 1 | 0.0 | 100 | unique | 0.705290 | 0.705290 | 0.300297 | 49,655.088 |
| main | 1 | 0.0 | 1000 | unique | 0.706148 | 0.706148 | 0.299293 | 45,647.280 |
| main | 1 | 0.0 | 5000 | unique | 0.707047 | 0.707047 | 0.298758 | 33,449.472 |
| main | 1 | 0.2 | 100 | unique | 0.553429 | 0.689848 | 0.315029 | 49,688.352 |
| main | 1 | 0.2 | 1000 | unique | 0.550571 | 0.689817 | 0.315005 | 45,647.280 |
| main | 1 | 0.2 | 5000 | unique | 0.552805 | 0.690178 | 0.315520 | 33,413.184 |
| main | 1 | 0.4 | 100 | unique | 0.400575 | 0.668126 | 0.336391 | 49,645.008 |
| main | 1 | 0.4 | 1000 | unique | 0.405555 | 0.673816 | 0.331320 | 45,566.640 |
| main | 1 | 0.4 | 5000 | unique | 0.402741 | 0.671334 | 0.333547 | 33,477.696 |
| main | 5 | 0.0 | 100 | unique | 0.853841 | 0.853841 | 0.153106 | 49,708.512 |
| main | 5 | 0.0 | 1000 | unique | 0.853017 | 0.853017 | 0.153234 | 45,707.760 |
| main | 5 | 0.0 | 5000 | unique | 0.854705 | 0.854705 | 0.152061 | 33,468.624 |
| main | 5 | 0.2 | 100 | unique | 0.664156 | 0.832467 | 0.174574 | 49,724.640 |
| main | 5 | 0.2 | 1000 | unique | 0.664427 | 0.831558 | 0.175253 | 45,674.496 |
| main | 5 | 0.2 | 5000 | unique | 0.664345 | 0.832450 | 0.174659 | 33,453.504 |
| main | 5 | 0.4 | 100 | unique | 0.482583 | 0.802174 | 0.205532 | 49,714.560 |
| main | 5 | 0.4 | 1000 | unique | 0.486090 | 0.802804 | 0.204807 | 45,687.600 |
| main | 5 | 0.4 | 5000 | unique | 0.482278 | 0.803271 | 0.204016 | 33,516.000 |
| main | 15 | 0.0 | 100 | unique | 0.955313 | 0.955313 | 0.050653 | 49,783.104 |
| main | 15 | 0.0 | 1000 | unique | 0.955667 | 0.955667 | 0.049741 | 45,765.216 |
| main | 15 | 0.0 | 5000 | unique | 0.955484 | 0.955484 | 0.049787 | 33,497.856 |
| main | 15 | 0.2 | 100 | unique | 0.756427 | 0.943452 | 0.062871 | 49,778.064 |
| main | 15 | 0.2 | 1000 | unique | 0.754612 | 0.943871 | 0.061772 | 45,754.128 |
| main | 15 | 0.2 | 5000 | unique | 0.755108 | 0.943666 | 0.061952 | 33,430.320 |
| main | 15 | 0.4 | 100 | unique | 0.557582 | 0.926261 | 0.079763 | 49,792.176 |
| main | 15 | 0.4 | 1000 | unique | 0.556241 | 0.925610 | 0.079613 | 45,778.320 |
| main | 15 | 0.4 | 5000 | unique | 0.555045 | 0.926269 | 0.079337 | 33,287.184 |
| control: no_attack_baseline | 15 | 0.0 | 100 | unique | 0.988384 | 0.988384 | 0.018666 | 0.000 |
| control: no_attack_worst | 1 | 0.4 | 5000 | unique | 0.406062 | 0.674463 | 0.330103 | 0.000 |
| control: repeated_benign_worst | 1 | 0.4 | 5000 | repeated | 0.345137 | 0.575279 | 0.425083 | 33,466.608 |

Note: The 27 main cells use 10000 attack RPAs/min and unique benign addresses. The three controls are the no-attack baseline, the no-attack adverse boundary, and the attacked repeated-benign adverse boundary. Observation loss is applied by the simulator after event generation and before scheduling; it is a controlled proxy for missed observations, not measured BLE packet loss. Generated-but-unobserved legitimate events remain in the offered denominator.

# Supplementary Table S4

Supplementary Table S4. Twenty-seed confirmation of the preregistered adaptive local-search union.

| Δ (s) | Rank | Strategy | n | Attack AES/min | 95% CI | Mean/cap | Legit rate | False defer | Removable fraction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | pool 224, r=1, p=120 | 20 | 146,162.016 | 145,986.621–146,337.411 | 0.966680 | 0.640442 | 0.360126 | 0.000000 |
| 0 | 2 | pool 256, r=1, p=105 | 20 | 146,162.016 | 145,986.621–146,337.411 | 0.966680 | 0.640442 | 0.360126 | 0.000000 |
| 0 | 3 | pool 256, r=1, p=120 | 20 | 146,162.016 | 145,986.621–146,337.411 | 0.966680 | 0.640442 | 0.360126 | 0.000000 |
| 0 | 4 | pool 256, r=1, p=135 | 20 | 146,162.016 | 145,986.621–146,337.411 | 0.966680 | 0.640442 | 0.360126 | 0.000000 |
| 0 | 5 | pool 256, r=2, p=120 | 20 | 146,162.016 | 145,986.621–146,337.411 | 0.966680 | 0.640442 | 0.360126 | 0.000000 |
| 0 | 6 | pool 320, r=1, p=105 | 20 | 146,162.016 | 145,986.621–146,337.411 | 0.966680 | 0.640442 | 0.360126 | 0.000000 |
| 0 | 7 | pool 320, r=1, p=120 | 20 | 146,162.016 | 145,986.621–146,337.411 | 0.966680 | 0.640442 | 0.360126 | 0.000000 |
| 0 | 8 | pool 320, r=1, p=135 | 20 | 146,162.016 | 145,986.621–146,337.411 | 0.966680 | 0.640442 | 0.360126 | 0.000000 |
| 0 | 9 | pool 320, r=1, p=75 | 20 | 146,162.016 | 145,986.621–146,337.411 | 0.966680 | 0.640442 | 0.360126 | 0.000000 |
| 0 | 10 | pool 320, r=1, p=90 | 20 | 146,162.016 | 145,986.621–146,337.411 | 0.966680 | 0.640442 | 0.360126 | 0.000000 |
| 0 | 11 | unique | 20 | 45,765.216 | 45,606.392–45,924.040 | 0.302680 | 0.955667 | 0.049741 | 0.000000 |
| 60 | 1 | pool 320, r=1, p=75 | 20 | 146,196.288 | 146,018.171–146,374.405 | 0.966907 | 0.674902 | 0.325627 | 0.970628 |
| 60 | 2 | pool 320, r=1, p=135 | 20 | 146,195.280 | 146,015.168–146,375.392 | 0.966900 | 0.675190 | 0.325339 | 0.970628 |
| 60 | 3 | pool 320, r=1, p=90 | 20 | 146,193.264 | 146,014.002–146,372.526 | 0.966887 | 0.674165 | 0.326364 | 0.970629 |
| 60 | 4 | pool 320, r=1, p=105 | 20 | 146,192.256 | 146,012.078–146,372.434 | 0.966880 | 0.673834 | 0.326695 | 0.970628 |
| 60 | 5 | pool 320, r=1, p=120 | 20 | 146,192.256 | 146,012.078–146,372.434 | 0.966880 | 0.673232 | 0.327297 | 0.970628 |
| 60 | 6 | pool 256, r=2, p=120 | 20 | 129,012.912 | 128,998.080–129,027.744 | 0.853260 | 0.929699 | 0.075611 | 0.974394 |
| 60 | 7 | pool 256, r=1, p=105 | 20 | 129,001.824 | 128,989.618–129,014.030 | 0.853187 | 0.928646 | 0.076663 | 0.974396 |
| 60 | 8 | pool 256, r=1, p=135 | 20 | 128,980.656 | 128,961.472–128,999.840 | 0.853047 | 0.928646 | 0.076663 | 0.974401 |
| 60 | 9 | pool 256, r=1, p=120 | 20 | 128,962.512 | 128,939.305–128,985.719 | 0.852927 | 0.928646 | 0.076663 | 0.974403 |
| 60 | 10 | pool 224, r=1, p=120 | 20 | 112,821.408 | 112,799.312–112,843.504 | 0.746173 | 0.928646 | 0.076663 | 0.977607 |
| 60 | 11 | unique | 20 | 45,758.160 | 45,597.956–45,918.364 | 0.302633 | 0.928646 | 0.076663 | 0.000000 |

Note: The confirmation union contains the mechanically selected top five, the historical reference, five single-axis neighbors, and the unique-address baseline after deduplication. The complete 168-strategy, five-seed screening table is supplied as adaptive_local_screen_complete.csv. Overlapping confidence intervals identify a tied damaging local region; the search does not establish a global or unique attacker optimum. The cap is 151,200 AES-equivalent attempts/min for the stated modeled parameters.